Remote Controller Compliance Documentation

Platform: Quadcopter (Pixhawk flight controller, Jetson Orin Nano companion computer)

Firmware Environment: ArduPilot / ArduCopter 4.7.0

Transmitter: RadioMaster Pocket (M2) — EdgeTX, internal ExpressLRS 2.4 GHz module

Team: Team Inspiration / ASTA — RobotX 2026 UAV segment

This document outlines the control system description and the testing scenarios required to demonstrate remote controller operational compliance for the UAV.

# 1.  Control System Description

The UAV control system uses a RadioMaster Pocket transmitter running EdgeTX with an internal ExpressLRS 2.4 GHz radio module, paired to a RadioMaster RP3 ExpressLRS 2.4 GHz nano receiver on the aircraft. The receiver delivers pilot commands to the Pixhawk flight controller over the SBUS protocol.

The vehicle operates on ArduPilot firmware and features a Jetson Orin Nano as the companion computer for mission execution. Although the UAV is designed for autonomous operations, the pilot retains full authority to manually take over control at any time to ensure operational safety. Manual takeover acts through the control link alone and does not depend on the ground control station, the telemetry link, or the companion computer.

## Control link specification

| Transmitter | RadioMaster Pocket (M2) — EdgeTX firmware |
| --- | --- |
| Receiver | RadioMaster RP3 ExpressLRS 2.4 GHz nano |
| Radio protocol | ExpressLRS, 2.4 GHz ISM band, frequency hopping |
| Transmit power | 250 mW (24 dBm) EIRP |
| Receiver output | SBUS to the flight controller (RC_PROTOCOLS = 1) |
| Failsafe threshold | Throttle PWM below 975 (FS_THR_VALUE = 975) |

# 2.  Switch Mapping

The following remote controller channels have been configured within the system to meet safety standards. The mapping of auxiliary switches may change in the future, excluding the channels mapped for throttle, roll, pitch and yaw.

| Channel (RC) | Function | Description & ArduPilot Parameter |
| --- | --- | --- |
| Channel 1 – 4 | Roll, Pitch, Throttle, Yaw | Basic flight movement controls (manual RC input). RCMAP_ROLL = 1, RCMAP_PITCH = 2, RCMAP_THROTTLE = 3, RCMAP_YAW = 4 |
| Channel 6 | Emergency Motor Stop (Arm / Disarm) | Instantly cuts power to all motors in the event of an emergency. Disarming removes all motor output immediately, regardless of throttle stick position. RC6_OPTION = 154 |
| Channel 7 | Return to Launch | Commands an immediate Return to Launch at any point in the mission. RC7_OPTION = 4 |
| Channel 8 | Flight Mode Switch | Toggles between flight modes such as Stabilize, Alt Hold, Loiter and Brake. FLTMODE_CH = 8 |
| Rudder stick | Arm / Disarm | Secondary arm and disarm method held on the yaw stick. ARMING_RUDDER = 2 |

## Flight modes on the mode switch

| Switch position | Flight mode | Parameter |
| --- | --- | --- |
| Position 1 | Brake — halt and hold position | FLTMODE1 = 17 |
| Position 2 | Stabilize — manual attitude | FLTMODE2 = 0 |
| Position 3 | Stabilize — manual attitude | FLTMODE3 = 0 |
| Position 4 | Alt Hold — altitude hold, manual position | FLTMODE4 = 2 |
| Position 5 | Stabilize — manual attitude | FLTMODE5 = 0 |
| Position 6 | Loiter — altitude and position hold | FLTMODE6 = 5 |

# 3.  Proof of Compliance Scenarios (File / Photo / Video)

To satisfy the committee's requirements, the following documentation demonstrates that the remote safety systems function correctly. All three scenarios have been verified.

## A.  Manual Takeover

This proves that the vehicle can be instantly retrieved from autonomous mode if an anomaly occurs with the sensors or the companion computer.

- Video Scenario: The UAV is flying under autonomous control. The pilot then toggles the flight mode switch on channel 8 of the RadioMaster Pocket to a pilot-controlled mode such as Loiter or Alt Hold. (Attached in Video/Pilot Takeover Test.mp4)

- Compliance Indicator: The Ground Control Station display must show a rapid change in the flight mode, and the flight logs must record the immediate transition to manual control. The mode change is processed on the next control-loop iteration, under one second, and is independent of the ground station and telemetry link.

## B.  Emergency Motor Stop Test

This function serves as the ultimate hardware safety layer if the UAV flies out of control.

- Video Scenario: The UAV is armed and the throttle is raised slightly so that all four motors are spinning but the aircraft is not hovering. The pilot then activates the emergency motor stop switch on channel 6 of the RadioMaster Pocket. (Attached in Video/Motor Emergency Switch.mp4)

- Compliance Indicator: All motors must stop spinning instantaneously, even if the throttle stick on the remote controller remains in the raised position. The flight log must record the disarm event at the moment the switch is activated.

## C.  Radio Loss Failsafe

This proves that the vehicle will automatically execute safety procedures when the connection to the pilot is lost.

- Video Scenario: The UAV is armed and connected to the Ground Control Station. The pilot deliberately powers off the RadioMaster Pocket transmitter. (Attached in Video/RC Link Loss Test.mp4)

- Compliance Indicator: The Ground Control Station display must immediately show a “RADIO FAILSAFE” warning as the throttle PWM signal drops below the 975 threshold (FS_THR_VALUE). After approximately one second below the threshold the flight mode must automatically switch to Return to Launch (FS_THR_ENABLE = 1). If the UAV is already on the ground when the failsafe triggers, it disarms instead of launching.

Any change to the control system, switch mapping, failsafe configuration or declared transmit power will be reflected in this document and declared before further flight activity.