UAV Emergency Behavior & Failsafe Documentation

Platform: Quadcopter (Pixhawk flight controller, Jetson Orin Nano companion computer)

Firmware Environment: ArduPilot / ArduCopter 4.7.0

Team: Team Inspiration / ASTA — RobotX 2026 UAV segment

Status: All emergency behaviours listed below have been verified in flight

# 1.  Loss of Link with Remote Controller (Radio Failsafe)

  - Detection Method: The system monitors the primary throttle channel for signal loss. The receiver delivers control frames over SBUS; loss of valid frames or a throttle value below the configured threshold is treated as a radio failsafe.

  - Trigger Threshold (FS_THR_VALUE): The failsafe triggers if the PWM signal drops to 975.

  - Timeout: The signal must remain below the threshold for approximately 1.0 second before the failsafe executes.

  - Failsafe Action (FS_THR_ENABLE): The action is set to 1, which commands the UAV to execute a Return to Launch (RTL). If the UAV is already on the ground when the failsafe triggers, it disarms instead of launching.

  - Return Altitude (RTL_ALT_M): On RTL the UAV climbs to the configured return altitude of 3 metres if it is below it, or maintains its current altitude if higher, before returning to the recorded launch point.

  - Return Sequence (RTL_LOIT_TIME, RTL_ALT_FINAL_M): On arrival the UAV holds for 5 seconds over the launch point, then descends and lands. Final descent is at 0.5 m/s below 10 metres (LAND_SPD_MS, LAND_ALT_LOW_M). Touchdown detection disarms the motors.

  - Recovery: Restoring the control link does not cancel the return. The pilot must move the flight-mode switch to resume manual control.

  - Crash Check (FS_CRASH_CHECK): Crash checking is enabled (1) to detect physical impacts and automatically shut down the motors if necessary.

# 2.  Low Battery Scenarios

  - Battery Monitoring (BATT_MONITOR): Set to 3, monitoring pack voltage. Battery capacity (BATT_CAPACITY) is configured for a 6,000 mAh 6S lithium polymer pack, 22.2 V nominal and 25.2 V maximum.

  - Voltage Source (BATT_FS_VOLTSRC): Set to 0, using measured pack voltage. Failsafe thresholds are evaluated against the live voltage under load.

  - Stage 1: Low Battery Warning

    - Voltage Threshold (BATT_LOW_VOLT): Triggers at 21.6 Volts (3.60 V per cell).

    - Timeout (BATT_LOW_TIMER): The voltage must remain low for 10 seconds to trigger, preventing a momentary current spike from commanding a return.

    - Action (BATT_FS_LOW_ACT): Set to 2 (RTL), meaning the UAV will abort the current mission and autonomously return home.

  - Stage 2: Critical Battery Warning

    - Voltage Threshold (BATT_CRT_VOLT): Triggers at 19.8 Volts (3.30 V per cell), with no timer.

    - Action (BATT_FS_CRT_ACT): Set to 2 (RTL). The UAV abandons the current task and returns to the launch point, descending and landing on arrival.

# 3.  Task / Mission Abort & EKF Failsafes

  - Manual Takeover: Moving the flight-mode switch to any new position immediately returns control to the pilot and terminates the active mission. The change is processed on the next control-loop iteration, under one second, and acts through the control link alone — it does not depend on the ground station or the telemetry link.

  - Dedicated Return Switch (RC7_OPTION): Set to 4. A dedicated transmitter switch commands Return to Launch at any point in the mission.

  - Available Flight Modes: BRAKE (halt and hold position), STABILIZE (manual attitude), ALT_HOLD (altitude hold with manual position, requiring no position estimate), LOITER (altitude and position hold), RTL and LAND.

  - EKF Failsafe Action (FS_EKF_ACTION): Set to 1 (Land). If the Extended Kalman Filter detects critical navigation sensor variance exceeding the 0.8 threshold (FS_EKF_THRESH), the UAV abandons autonomous navigation and lands immediately. ALT_HOLD remains available to the pilot because it requires no position estimate.

  - Dead Reckoning (FS_DR_ENABLE): Set to 2 with a 30 second timeout (FS_DR_TIMEOUT). On loss of position the UAV dead-reckons for up to 30 seconds before further failsafe action is taken.

  - Return to Launch Altitude (RTL_ALT_M): Configured to 3 metres. Upon an RTL abort command, the UAV climbs to this altitude, or maintains its current altitude if higher, before returning home.

  - Ground Control Station Failsafe (FS_GCS_ENABLE): Set to 1 with a 5 second timeout (FS_GCS_TIMEOUT). Loss of the telemetry link commands a Return to Launch during autonomous flight. FS_OPTIONS is set to 16, so the UAV continues without interruption while the pilot is flying in a pilot-controlled mode — control is not taken from a pilot who still has a good control link and eyes on the vehicle.

  - Vibration Failsafe (FS_VIBE_ENABLE): Enabled (1). On detection of excessive vibration the UAV switches to a vibration-resistant position control mode.

  - Emergency Stop (ARMING_RUDDER): Set to 2. A rudder input arms and disarms the UAV; disarming cuts all motor output immediately. The UAV also disarms automatically 10 seconds after landing (DISARM_DELAY).

# 4.  Geofence Boundary Reached

  - Geofence Status (FENCE_ENABLE): The geofence is enabled (1) and is always active rather than armed at takeoff (FENCE_AUTOENABLE is 0).

  - Fence Type (FENCE_TYPE): Set to 7 (Altitude ceiling, circular and polygon limits all enforced).

  - Maximum Radius (FENCE_RADIUS): Hard limit set to 30 metres from the home point.

  - Maximum Altitude (FENCE_ALT_MAX): Hard limit set to 30 metres, below the 60 metre competition ceiling. A floor is also set at −10 metres (FENCE_ALT_MIN).

  - Fence Margin (FENCE_MARGIN): A 2 metre margin is configured, which allows the flight controller to begin intervening just before the UAV reaches the hard limit.

  - Active Avoidance (AVOID_ENABLE): Set to 2 with a 4 metre margin (AVOID_MARGIN). In position-controlled modes the UAV decelerates at up to 3 m/s² and slides along the boundary rather than crossing it (AVOID_BEHAVE is 0), so the boundary is defended before any breach occurs.

  - Failsafe Action (FENCE_ACTION): Set to 1 (RTL, or Land if a return is not possible), which halts the UAV's outward progress and returns it home to prevent a flyaway if the boundary is breached.

  - Boundary Notification (FENCE_NTF_FREQ): Enabled (1). The ground control station receives an explicit boundary and breach message.